CP660

LTE Indoor CPE

3GPP Release 10/11

CAT6

802.11a/b/g/n/ac

VoIP or VoLTE

The CP660 from BTI WIRELESS is a high-performance indoor LTE CPE, designed for 802.11a/b/g/n/ac dual band Wi-Fi access. The product provides optimized high throughput and a wide area coverage network for residential, commercial and enterprise users, supporting multiple services. Customers enjoy reliable dual-band Wi-Fi AP functions, broadband access and hot-spot Wi-Fi connections with the CP660.

| RF SPECIFICATIONS |  |  |
| --- | --- | --- |
| Supports Bands | Option 1 | LTE: B1/3/7/8/20/28/38/40/41/42/43 |
|  | Option 2 | LTE: B7/38/41 |
| Carrier Aggregation (CA) | Inter/ Intra 2CA |  |
| LTE Antenna | 4 built-in antennas |  |
| MIMO | CA: Intra- or Inter-band DL 2 CA (2 x 2 MIMO 64 QAM) DL 4 x 4 MIMO 64 QAM UL 64 QAM |  |
| Output Power @ 25oC/ 77oF | 23 ± 2dBm |  |
| LTE Antenna Gain | 700 MHz ~ 1 GHz | 3 dBi |
|  | 1.5 GHz ~ 2.7 GHz | 5 dBi |
|  | 3.4 GHz ~ 3.8 GHz | 5 dBi |
| Wi-Fi Frequency | 2.4 GHz/ 5 GHz |  |
| Wi-Fi Antenna | Built-in high-gain Wi-Fi antennas (2.4 GHz/ 5 GHz) |  |
| Wi-Fi Output Power | 2.4 GHz: 19dBm |  |
|  | 5 GHz: 16dBm |  |
| Wi-Fi Channel BW | 20 MHz & 40 MHz & 80 MHz |  |
| Wi-Fi Antenna Gain | 2.4 GHz: 3 dBi |  |
|  | 5 GHz: 3 dBi |  |

| ## HARDWARE SPECIFICATIONS |  |
| --- | --- |
| LTE Standard | 3GPP Release 10/11, CAT 6 |
| Flash | 2 GB |
| RAM | 2 GB |
| Ethernet LAN Port | 2 gigabit RJ45 LAN port 10/100/1000M auto-sensing, auto-MDI/ MDIX |
| Voice Port (Optional) | 1 RJ11 |
| LED | 8 LED Indicators (PWR, Wi-Fi, TEL, NET, RF Signal x 4) |
| SIM | 1 USIM/ SIM (3FF) |
| Reset/Reboot Button | Tact Switch |
| Antennas | 4 built-in antennas |
| Dimensions | 204.5 mm x 169 mm x 60 mm ¦ 8.03 in x 6.65 in x 2.36 in |
| Weight | < 500 g ¦ 1.1 lbs. |
| Power Consumption | < 12W |
| Power Adapter | AC 100 ~ 240V DC 12V 2A |

| ENVIRONMENT SPECIFICATIONS |  |  |
| --- | --- | --- |
| Temperature | Operating | -10 ℃ ~ 55 ℃ ¦ 14 °F ~ 131 °F |
|  | Storage | -40 ℃ ~ 85 ℃ ¦ -40 °F ~ 185 °F |
| Humidity | Operating | 5 ~ 95% (Non-condensing) |
|  | Storage | 5 ~ 95% (Non-condensing) |

## TOPOLOGY

| SOFTWARE SPECIFICATIONS |  |
| --- | --- |
| Peak Throughput | TDD DL 220 Mbps UL15 Mbps FDD DL 300 Mbps UL75 Mbps |
| Operation Mode | NAT mode |
|  | Route Mode |
|  | Bridge Mode |
| Device Management | TR069 |
|  | SNMP |
|  | WebUI |
|  | Software Upgrade - FOTA |
| Network Protocols and Features | Multi-APN |
|  | NAT/NATP |
|  | SNTP/NITZ |
|  | Static Routing |
|  | Virtual server and DMZ |
|  | DNS relay and DDNS |
|  | IGMP snooping and IGMP proxy |
|  | DHCP server |
|  | IPV4/IPv6 Dual Stack |
|  | UPnP |
|  | Parents control |
| VPN | IPsec L2TP over IPsec GRE over IPsec GRE L2/L3 PPTP L2TP V2/V3 |
| Security and Firewall | Firewall |
|  | MAC address filtering |
|  | IP address filtering |
|  | URL filtering |
|  | DDOS |
| VoIP/ VoLTE Functions | Call waiting |
|  | Call hold/ resume |
|  | Call forward on busy/ no answer |
|  | Call transfer blind transfer/ attended transfer/ early attended transfer |
|  | 3-way conferencing |
|  | Caller ID display/ blocking |
|  | DTMF |
|  | FAX/ modem |
| Wi-Fi Functions | Hide SSID broadcasting |
|  | Encryption: WPA-PSK (TKIP), WPA-PSK (AES), WPA2-PSK (TKIP), WPA2-PSK (AES) |
|  | Maximum users: 64 |
|  | AP Isolation |
|  | Guest network |

Contact Us

www.btiwireless.com

sales@btiwireless.com