CP680

LTE Outdoor CPE

3GPP Release 10/11

CAT6

All LTE Bands

PoE

IP67

The CP680 from BTI WIRELESS is a high-performance outdoor LTE CPE. The product provides a high throughput data transmission network for residential, commercial and enterprise users. Customers enjoy an easy plug & play broadband access solution and large area coverage for various network service scenarios.

| ## CELLULAR SPECIFICATIONS |  |  |
| --- | --- | --- |
| LTE Standard | 3GPP Release 10/ 11, CAT 6 |  |
| Peak Throughput | FDD DL 300Mbps; UL 75Mbps |  |
|  | TDD DL 220Mbps; UL 15Mbps (Config 2:7) |  |
| LTE Bands* | Option 1 | B1/3/7/8/(17)/20/(28)/38/(40)/41/42/43/48 |
|  | Option 2 (Prototype) | B2/4/5/8/12/14/17/25/26/38/41/42/43/48/66 |
|  | Option 3 | B42/43/48 |
|  | Option 4 | B7/38/41 |
| Carrier Aggregation (CA) and MIMO | CA: DL Intra/Inter 2CA |  |
|  | MIMO: DL 2 x 2 MIMO/ 4 X 4 MIMO |  |
| Tx / Rx | 1Tx / 4Rx |  |
| Transmit Power | Power Class 3 (23 ± 2dBm) |  |
| LTE Antenna | 4 directional antennas |  |
| LTE Antenna Gain | Option 1 | 700 MHz ~ 1 GHz: 6 dBi 1.7 GHz ~ 2.1 GHz: 7 dBi 2.3 GHz ~ 2.7 GHz: 9 dBi 3.4 GHz ~ 3.8 GHz: 8 dBi |
|  | Option 2 | 700 MHz ~ 1 GHz: 6 dBi 1.7 GHz ~ 2.1 GHz: 7 dBi 2.3 GHz ~ 2.7 GHz: 9 dBi 3.4 GHz ~ 3.8 GHz: 8 dBi |
|  | Option 3 | 3.4 GHz ~ 3.8 GHz: 15 dBi |
|  | Option 4 | 2.5 GHz ~ 2.7 GHz: 13 dBi |

*The frequency bands in () are not included in the conventional sample.

## TOPOLOGY

| HARDWARE SPECIFICATIONS |  |
| --- | --- |
| Fixed Interfaces | 1 x RJ45 10/ 100/ 1000Mb/s |
|  | 1 x SIM slot (1.8V/ 3.0V 2FF) |
| LED Indicator | 4 LED indicators (PWR, NET, SIM, RF signal) |
| Reset | Pin Hole |
| Power Supply | 24V non-standard POE default |
|  | IEEE 802.3at compatible POE customized |
| Power Consumption | < 8W |
| Power Adapter | Input: AC 90 ~ 240V |
|  | Output: DC 24V/ 0.5A |
| Water and Dustproof | IP67 |
| Dimensions and Mounting | Dimensions: 288 x 288 x 82 mm ¦ 11.3 x 11.3 x 3.2 in |
|  | Weight: < 2 kg ¦ 4 lbs |
|  | Mounting: rail and wall mounting |
| Temperature and Humidity | Operating: -40 ℃ ~ +55 ℃ ¦ -40 °F ~ +131 °F |
|  | Storage: -40 ℃ ~ +85 ℃ ¦ -40 °F ~ +185 °F |
|  | Humidity: 5% ~ 95% |

| SOFTWARE SPECIFICATIONS |  |
| --- | --- |
| Operation Mode | NAT mode |
|  | Route Mode |
|  | Bridge Mode |
| Device Management | TR069 |
|  | SNMP |
|  | Web UI |
|  | Software upgrade- FOTA |
|  | USIM PIN management |
| Network Protocols and Features | Multi-APN |
|  | NAT/NATP |
|  | SNTP/NITZ |
|  | Static Routing |
|  | Virtual server and DMZ |
|  | DNS relay and DDNS |
|  | IGMP snooping and IGMP proxy |
|  | DHCP server |
|  | IPV4/IPv6 Dual Stack |
|  | UPnP |
|  | Parents control |
| VPN | IPsec |
|  | PPTP |
|  | GRE (L2/L3) |
|  | L2TP v2/v3 |
|  | GRE over IPsec |
|  | L2TP over IPsec |
| Security and Firewall | MAC address filtering |
|  | IP address filtering |
|  | URL filtering |
|  | DDOS |

Contact Us

www.btiwireless.com

sales@btiwireless.com