CP880

LTE Outdoor CPE

3GPP Release 12

CAT12

All LTE Bands

PoE

IP67

The CP880 from BTI WIRELESS is a premium outdoor LTE CPE. The product provides a high throughput data transmission network for residential, commercial and enterprise users. Customers enjoy an easy plug & play broadband access solution and large area coverage for various network service scenarios.

| RF SPECIFICATIONS |  |  |
| --- | --- | --- |
| Supports Bands* | Option 1 | B1/3/7/8/(17)/20/(28)/38/(40)/41/42/43/48 |
|  | Option 2 (Prototype) | B2/4/5/8/12/14/17/25/26/38/41/42/43/48/66 |
|  | Option 3 | B42/43/48 |
|  | Option 4 | B7/38/41 |
| Peak Throughput | TDD DL 560Mbps; UL 30Mbps (Config 2:7) FDD DL 800Mbps; UL 150Mbps |  |
| Carrier Aggregation (CA) | DL Inter/Intra 2CA/3CA/4CA UL Inter/Intra 2CA |  |
| MIMO | DL 2 x 2 (4 CA 256 QAM) DL 4 x 4 (2 CA 64 QAM) UL 2 x 2 64 QAM |  |
| Tx / Rx | 2 Tx / 4 Rx |  |
| Output Power @ 25 ℃ ¦ 77 ℉ | Max 23 ± 2dBm, configurable |  |
| Receiving Sensitivity | FDD | -100dBm/ 10 MHz @ QPSK |
|  | TDD | -97dBm/ 20 MHz @ QPSK |
| LTE Antenna | 4 directional antennas |  |
| LTE Antenna Gain | Option 1 | 700 MHz ~ 1 GHz: 6 dBi 1.7 GHz ~ 2.1 GHz: 7 dBi 2.3 GHz ~ 2.7 GHz: 9 dBi 3.4 GHz ~ 3.8 GHz: 8 dBi |
|  | Option 2 | 700 MHz ~ 1 GHz: 6 dBi 1.7 GHz ~ 2.1 GHz: 7 dBi 2.3 GHz ~ 2.7 GHz: 9 dBi 3.4 GHz ~ 3.8 GHz: 8 dBi |
|  | Option 3 | 3.4 GHz ~ 3.8 GHz: 16 dBi |
|  | Option 4 | 2.5 GHz ~ 2.7 GHz: 13 dBi |

*The frequency bands in () are not included in the conventional sample.

| ## HARDWARE SPECIFICATIONS |  |
| --- | --- |
| LTE Standard | 3GPP Release 12, CAT 12 |
| Flash | 2 GB |
| DDR | 2 GB |
| Power Supply | 24V Non-standard PoE default IEEE 802.3 at compatible PoE customized |
| Ethernet LAN Port/POE Input | 1 x RJ45 10/100/1000Mb/s |
| Water and Dust proof | IP67 |
| LED | 4 LED Indicators (PWR, NET, SIM, RF Signal) |
| SIM | 1 x SIM Slot (1.8V/3.0V 2FF) |
| Reset | Pin Hole |
| Antenna | 4 Built-in directional antennas |
| Dimensions | 288 x 288 x 82 mm ¦ 11.3 x 11.3 x 3.2 in |
| Weight | ODU < 2 kg ¦ 4.41 lbs |
| Power Consumption | < 8W |
| Power Adapter | Input: AC 90 ~ 240V Output: DC 24V/ 0.5A |

| ENVIRONMENT SPECIFICATIONS |  |  |
| --- | --- | --- |
| Temperature | Operating | -40 ℃ ~ +55 ℃ ¦ -40 °F ~ +131 °F |
|  | Storage | -40 ℃ ~ +85 ℃ ¦ -40 °F ~ +185 °F |
| Humidity | Operating | 5 ~ 95% |
|  | Storage | 5 ~ 95% |

## TOPOLOGY

| SOFTWARE SPECIFICATIONS |  |
| --- | --- |
| Operation Mode | NAT mode |
|  | Route Mode |
|  | Bridge Mode |
| Device Management | TR069 |
|  | SNMP |
|  | WebUI |
|  | Software Upgrade - FOTA |
| Network Protocols and Features | Multi-APN |
|  | NAT/NATP |
|  | SNTP/NITZ Static Routing |
|  | Virtual server and DMZ |
|  | DNS relay and DDNS |
|  | IGMP snooping and IGMP proxy |
|  | DHCP server |
|  | IPV4/IPv6 Dual Stack |
|  | UPnP |
|  | Parents control |
| VPN | IPSec |
|  | PPTP |
|  | L2TP V2/V3 |
|  | GRE L2/L3 |
|  | L2TP over IPsec |
|  | GRE over IPsec |
| Security and Firewall | MAC Address Filtering |
|  | IP Address Filtering |
|  | URL Filtering |
|  | DDOS |

Contact Us

www.btiwireless.com

sales@btiwireless.com