CP880I

LTE Outdoor Industrial CPE

CAT12

IP67

IEEE802.11a/b/g/n/ac Wi-Fi

The CP880I from BTI WIRELESS is an industrial CPE, providing sustainable and durable LTE service with higher data transmission speed for video and other bandwidth-intensive applications. The industrial CP880I CPE has a special anti-vibration feature, which qualifies it for extreme environments where it can work in a temperature range from -40 °C to 70 °C. It is an ideal industrial CPE for outdoor scenarios with crucial industrial requirements, such as Railway, Transportation, Manufacturing, Mining, Oil and Gas, etc.

The CP880I is integrated with multiple LTE-A modules, which make it possible to connect with multiple bands of private LTE, including uncommon frequencies. The CP880I also provides accurate, real-time location information via the built-in GNSS receiver, able to collaborate with GPS, GLONASS, BEIDOU and Galileo constellations.

| ## CELLULAR SPECIFICATIONS |  |  |
| --- | --- | --- |
| Peak Throughput | FDD: DL 400 Mbps UL 150 Mbps TDD: DL 280 Mbps UL 30 Mbps |  |
| LTE Bands | CP880I-A | Band 1/3 |
|  | CP880I-B | Band 2/4/5/8/12/13/14/25/26/38/41/42/43/48/53/66 |
|  | CP880I-C | Band 46/240 |
| Carrier Aggregation and MIMO | DL: 2 CA Intra- or Inter-band, 2 x 2 MIMO, 256 QAM UL: 2 CA or 2 x 2 MIMO, 64 QAM |  |
| Tx / Rx | 2Tx / 2Rx |  |
| EIRP | < 23dBm (Include antenna) |  |

## TOPOLOGY

| HARDWARE SPECIFICATIONS |  |
| --- | --- |
| Fixed Interfaces | 1 x M12 for Power 2 x M12 for 10/100/1000 Mbps compatible GE 1 x M12 for RS232 + 2 x DI + 2 x DO 1 x M12 for RS485 2 x Nano SIM slot (4FF, Dual SIM Single Standby) |
| Antenna Ports | 2 x TNC female for cellular antennas 1 x TNC female for GNSS antenna 2 x TNC female for Wi-Fi antennas |
| GNSS | Supports GPS, GLONESS, BEIDOU and Galileo Constellation Acquisition sensitivity: 148dBm (GPS at cold start) Tracking sensitivity: 167dBm (GPS in continuous mode) |
| LED Indicator | 1 x PWR indicates Power Status 1 x NET indicates Network Status 1 x RSSI LED indicates SIM/Signal status |
| Button | 1 x Reboot or Restore Button |
| Power Consumption | < 13W |
| Power supply | 9~36V DC |
| Dimensions | 250 x 171 x 61mm ¦ 9.84 x 6.73 x 2.40 in |
| Weight | ≈ 2 kg ¦ 4.4 lbs |
| EMC Compliance | EN55032 (CE&RE) Class B IEC61000-4-2(ESD) Level 4 IEC61000-4-3 (RS) Level 2 IEC61000-4-4 (EFT) Level 4 IEC61000-4-5 (Surge) Level 3 IEC61000-4-6 (CS) Level 2 |
| Environment Compliance | Cold: IEC60068-2-1 Dry heat: IEC 60068-2-2 Damp heat cyclic: IEC 60068-2-30 Change of temperature: IEC 60068-2-14 Protection Rating: EN60529, IP67 Shock and Vibration: EN50155 Free Fall: IEC60068-2-32 (1 meter height with package) Salt Mist: IEC 68-2-11, 72H |
| WAN | Multiple APN support, maximum 4 IP pass-through IPv4/IPv6 dual stack |
| LAN | VLAN 802.1Q DHCP Server DNS and DNS proxy DMZ Multicast/Multicast Proxy |
| Protocol | Routing: static routing |
| Device Management | TR069 SNMP V2 Web UI Software upgrade via WEB/ TR069/ FOTA USIM PIN management |
| Routing Mode | Route mode Bridge mode NAT mode Static route Port mirror and port forwarding IPv4/ IPv6 dual stack |
| VPN | PPTP GRE tunnel (L2/L3) L2TPv2 and L2TPv3 |
| Security | Firewall MAC address filtering IP address filtering URL filtering Access control HTTPS login from WAN Dos attack protection 2 level of user authority |
| Reliability | Watchdog for automatic recovery Auto-rollback to previous version when upgrade fails Dual WAN for link backup |

| ENVIRONMENT SPECIFICATIONS |  |  |
| --- | --- | --- |
| Temperature | Operating | -40 ℃ ~ +70 ℃ ¦ -40 °F ~ +158 °F |
|  | Storage | -40 ℃ ~ +85 ℃ ¦ -40 °F ~ +185 °F |
| Humidity | 5 ~ 95% non-condensing |  |

Contact Us

www.btiwireless.com

sales@btiwireless.com