CP980I

LTE Outdoor Industrial CPE

TR069

IP67

3GPP Release 14 CAT16 DL

Wi-Fi 6 (802.11ax)

The CP980I outdoor CPE from BTI WIRELESS provides LTE service in a ruggedized form factor. With an operating temperature range of -40 to +70 degree Celsius endurance, this product offers industrial-grade environmental qualifications while providing higher speeds data services for video and other bandwidth-intensive applications.

The CP980I has very strong anti-vibration ability, which qualifies it for extreme industrial environments and ideally suited for rail, transportation, mining, oil and gas, manufacturing, and other outdoor applications. The device can support multi-bands for LTE and can provide accurate real-time location information. The built-in GNSS receiver supports GPS, GLONASS, BEIDOU and Galileo constellations.

| ## CELLULAR SPECIFICATIONS |  |  |
| --- | --- | --- |
| Category | 3GPP Release 14 CAT16 DL |  |
| Peak Throughput | DL: 1Gbps (CAT16) UL: 150Mbps (CAT13) |  |
| Frequency Bands | LTE Bands | LTE-FDD： B1/B2/B3/B4/B5/B7/B8/B12/B13/B14/B17/B18/B19/ B20/B25/B26/B28/B29/ B30/B32/B66 LTE-TDD： B38/B39/B40/B41/B42/B43/B46(LAA)/B48(CBRS) |
|  | WCDMA | B1/B2/B3/B4/B5/B6/B8/B19 |
| Carrier Aggregation and MIMO | DL: 5CA intra or inter band/4x4MIMO/256QAM UL: 2CA intra band (CA_3C, CA_41C)/64QAM |  |
| Tx / Rx | 1Tx / 4Rx |  |
| Transmit Power | Power Class 3 (23±2dBm) |  |
| Wi-Fi | Supports 2.4GHz/5GHz dual band IEEE802.11 b/g/n/ac/ax Wi-Fi |  |

## TOPOLOGY

| HARDWARE SPECIFICATIONS |  |
| --- | --- |
| Chipset | Qualcomm IPQ6010 + SDX24 |
| Fixed Interfaces | 1 x M12 for Power supply 1 x M12 for 10/100/1000Mbps compatible GE port 1 x M12 for 2.5GHz GE port 1 x M12 for RS232 + 2 x DI + 2 x DO 1 x M12 for RS485 2 x Nano SIM Slot (4FF, Dual SIM Single Active) |
| Antenna Ports | 3 x TNC female for LTE antennas 1 x TNC female for LTE&GNSS antenna 2 x TNC female for Wi-Fi antennas |
| GNSS | Supports GPS, GLONESS, BEIDOU and Galileo |
| LED Indicator | 1 x Power indicator 1 x Network status indicator 1 x RSSI indicator |
| Button | 1 x Reboot or Restore Button |
| Power Consumption | < 18W |
| Operating Voltage | 9V ~ 36V DC |
| Dimensions | 250 x 171 x 61mm ¦ 9.84 x 6.73 x 2.40 in |
| Weight | ≈ 2 kg ¦ 4.41 lbs |

| SOFTWARE SPECIFICATIONS |  |  |
| --- | --- | --- |
| WAN | Multiple APN support, maximum 4 |  |
|  | Multi PDN |  |
|  | IPv4/IPv6 dual stack |  |
| LAN | VLAN 802.1Q |  |
|  | DHCP V4/V6 |  |
|  | DNS and DNS proxy |  |
|  | DMZ |  |
|  | UPNP |  |
|  | ALG |  |
|  | ARP/ICMP/IGMP |  |
|  | MAC address Filtering |  |
| Device Management | Version management: Image Checksum |  |
|  | HTTP/FTP Auto Upgrade |  |
|  | TR069 | Upload/ Download the ACS specified file |
|  |  | Download the configuration file |
|  |  | Configure & queries parameters |
|  |  | Remote upgrade |
|  |  | Remote debugging operations |
|  |  | Cycle monitor |
|  | SNMP |  |
|  | WebGUI/ CLI | Different level of user login priority and different limitation and display |
|  |  | Encryption backup current settings and restore the backup settings |
|  |  | Export current diagnosis results and operation logs |
|  |  | Statistics of LAN link status, transmit and receive traffic and up time |
|  | Diagnostics |  |
|  | USIM PIN management and card authentication |  |
|  | USIM activation |  |
|  | NAT/Bridge mode |  |
|  | Port mirror/Port forwarding |  |
| VPN and Routing | Static Routing |  |
|  | IPsec |  |
|  | PPTP |  |
|  | GRE Tunnel |  |
|  | L2TP v2/v3 |  |
| Security | Firewall |  |
|  | MAC address filtering |  |
|  | IP address filtering |  |
|  | URL filtering |  |
|  | Access control |  |
|  | HTTPS login from WAN |  |
|  | Dos attack protection |  |
|  | 2 level of user authority |  |

| ENVIRONMENT SPECIFICATIONS |  |  |
| --- | --- | --- |
| Temperature | Operating | -40 ℃ ~ +70 ℃ ¦ -40 °F ~ +158 °F |
|  | Storage | -40 ℃ ~ +85 ℃ ¦ -40 °F ~ +185 °F |
| Humidity | Operating | 10 ~ 90% |
|  | Storage | 5 ~ 95% |

Contact Us

www.btiwireless.com

sales@btiwireless.com