nCELL-M4460

5G Integrated Base Station

40W

4T4R

All-in-one Design

Clock Synchronization

The nCELL-M4460 from BTI WIRELESS is based on advanced multi-core ARM and FPGA solutions and adopts an integrated design method of 5G BBU and RRU. Based on a completely independent research and development protocol stack and system software, it realizes a complete 5G NR wireless access, which can quickly provide users with a reliable 5G wireless coverage network.

The nCELL-M4460 has the advantages of low power consumption, small size, convenient construction, etc., and is suitable for applications in many 5G vertical industries.

| ## SYSTEM FUNCTIONS |  |
| --- | --- |
| Standard | 3GPP R16 |
| Number of Cells | 1 x 4T4R or 2 x 2T2R |
| Cell Capacity | 400 RRC users per cell |
| Cell Throughput | DL 1.5 Gbps, UL 260 Mbps (DDDSU) DL 658 Mbps, UL 669 Mbps (DSUUU) |
| Operating Frequency Band | N38: 2570-2620 MHz N40: 2300-2400 MHz N41: 2496-2690 MHz N78: 3300-3600 MHz N77/N78: 3600-3800MHz Band Customization |
| Channel Bandwidth | 20MHz/40MHz/50MHz/60MHz/80MHz/100MHz |
| RF Power | 4*40W |
| Duplex Mode | TDD |
| Subcarrier | 30 kHz |
| Clock Synchronization Method | GPS, 1588V2 clock synchronization |
| Power Supply | DC -48V |
| Power Consumption | < 700W |

## HARDWARE INTERFACE

| ## HARDWARE INTERFACE |  |
| --- | --- |
| Fronthaul (Connect To Remote RRU)/Backhaul/ Cascade Interface | 10G SFP+ optical port |
| DEBUG/RGPS Interface | Cable port |
| Power Input | Waterproof aviation plug |
| Radio Frequency Interface ANT1 -ANT4 | 4.3-10 |
| GPS Antenna Connector | GPSN |

| STRUCTURE PARAMETERS |  |
| --- | --- |
| Total Weight | BBU: 6.3 kg ¦ 13.89 lbs RRU: 27.9 kg ¦ 61.5 lbs |
| Dimension | BBU: 378.5 x 278.5 x 80 mm ¦ 14.9 x 10.96 x 3.15 in RRU: 500 x 380 x 142 mm ¦ 19.7 x 14.96 x 5.59 in |
| Installation Method | Supports pole installation, hanging installation, wall installation |

| ENVIRONMENTAL SPECIFICATIONS |  |
| --- | --- |
| Protection Level | IP65 |
| Operating Temperature | -40 ℃ ~ +55 ℃ ¦ -40 ℉ ~ +131 ℉ |
| Working Humidity | 5% ~ 95% |
| Working Pressure | 70kPa ~ 106kPa |

Contact Us Today

www.btiwireless.com

sales@btiwireless.com