nCELL-RU4460

5G Remote Radio Unit

4T4R Digital Radios

40W (46dBm) Output Power

5GNR

IP67 Outdoor Rated

The RU4460 from BTI is a high output power radio frequency unit (RRU) for 5G NR outdoor base stations. It forms a complete 5G base station together with the baseband unit (BBU), which is used in mobile communication cell cellular networking scenarios.

The platform support fiber optic cable cellular technology with CPRI/eCPRI protocol.The Power amplifier technology uses DPD (digital pre-distortion) technology, compared with analog technology, power consumption has been significantly improved. The platform is ideal for underground tunnel and outdoor overlay deployments for cellular services.

## KEY FEATURES

- Support 4T4R, 5G NR Compliant - Up to 40W (46dBm) Output Power and 100MHz IBW - Supports Sub-6GHz TDD & FDD and External Alarm

| ## TECHNICAL SPECIFICATIONS |  |
| --- | --- |
| ## SYSTEM |  |
| Bandwidth per Channel (Downlink & Uplink) | ≤ 100 MHz (Contiguous) |
| Digital Bandwidth per Channel (Downlink & Uplink) | 3/5/10/20MHz LTE 20/40/50/60/80/100MHz NR |

| SUPPORTED BANDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Band Frequency | 3GPP Band | Downlink Frequency | Uplink Frequency | Max Bandwidth |
| 2570-2620 TDD | N38 | 2570-2620 | 2570-2620 | 100 |
| 2300-2400 TDD | N40 | 2300-2400 | 2300-2400 | 100 |
| 2496-2690 TDD | N41 | 2496-2690 | 2496-2690 | 100 |
| 3300-3600 TDD | N78 | 3300-3600 | 3300-3600 | 100 |
| 3600-3800 TDD | N78 | 3600-3800 | 3600-3800 | 100 |
| * More bands will be supported in the roadmap |  |  |  |  |

| INTERFACES |  |
| --- | --- |
| Antenna Interface (All bands) | 4.3-10 Female |
| Optical Connector Type | SFP+, Standard LC |
| Optical Transmission Rate | 9.8304GB/s |
| Optical Fiber Length | 1.4km / 10km / 30km 0.87mi / 6.21mi / 18.64mi |
| Dry Contact | 2 Inputs & 2 Outputs, NO and NC Mode |
| Maintenance Interface | Ethernet RJ45 |

| ELECTRICAL |  |
| --- | --- |
| Complies with | 3GPP TS36.106 ¦ 3GPP TS25.106 |
| EMC | EN 301489-1 / -50, EN 50121-4, EN 55032, EN 61000-4 series |
| Safety | EN 60950-1, EN 60950-22, EN 62368-1, EN 50385 |
| Maximum Power Consumption | 650W |
| Power Supply | 48VDC ± 20% |

| ENVIRONMENTAL |  |
| --- | --- |
| Mean Time Between Failure (MTBF) | > 100,000 hours |
| Operating Temperature | 40 °C ~ +55 °C ¦ -40 °F ~ +131 °F |
| Humidity | 5% to 100% (Non-Condensing) |
| Cooling | Passive |
| Installation | Wall or Pole |
| Ingress Protection Rating | IP67 (Outdoor) |
| Complies with | EN 300019-1-1, EN 300019-1-2, EN 300019-1-4 |

| MECHANICAL |  |
| --- | --- |
| Size (Width x Height x Depth) | 500 x 380 x 142 mm ¦ 19.7 x 14.96 x 5.59 in |
| Weight | 27.9 kg ¦ 61.5 lbs |

Contact Us Today

www.btiwireless.com

sales@btiwireless.com