CPX80I

5G Industrial Outdoor CPE

CAT20

5GNR/LTE/WCDMA

RS485/RS232

IP67

The CPX80I from BTI WIRELESS is a 5G Industrial Outdoor CPE. It supports various 4G/5G spectrums and ultra-high speed in a ruggedized form factor. This product offers industrial-grade environmental qualifications while providing high speed data services for the bandwidth-intensive applications. Up to IP67 with water-proof, shock-resistant, and lightning-proof, combined to ensure the quality of wireless communication under different outdoor environments.

## KEY FEATURES

- Qualcomm SDX62 chipset solution - Supports BOTH SA and NSA - Global/CBRS bands supported - Ruggedized and anti-vibration design - Powerful software features, supporting all router features - Supports Wi-Fi 6 - Web, TR069 and SNMP based device management - Supports 2.5G WAN/LAN & 1G LAN and diverse industry interfaces - IP67 rating for outdoor environment

## SYSTEM ELEMENTS

| ## HARDWARE SPECIFICATIONS |  |  |
| --- | --- | --- |
| 3GPP | 3GPP Rel-16 |  |
| Module | Quectel RM520N-GL |  |
| Cellular Bands | 5G NR | n1/n2/n3/n5/n7/n8/n12/n13/n14/n18/n20/n25/n26/n28/n29/n30/n38/n40/n41/n48/ n66/n70/n71/n75/n76/n77/n78/n79 |
|  | LTE | LTE-FDD:B1/B2/B3/B4/B5/B7/B8/B12/B13/B14/B17/B18/B19/B20/B25/B26/B28/B29/B30/B32/B66/B71 LTE-TDD: B34/B38/B39/B40/B41/B42/B43/B46(LAA)/B48 |
|  | WCDMA | B1/B2/B4/B5/B8/B19 |
| GNSS | GPS/GLONASS/BeiDou/Galileo |  |
| WiFi | Support 2.4GHz/5GHz dual band and IEEE802.11 b/g/n/ac/ax Wi-Fi |  |
| Network Mode | Supports SA/NSA |  |
| External Port | 1 x M12 for power supply 1 x M12 for 10/100/1000Mbps compatible GE port 1 x M12 for 2.5GHz GE port 1 x M12 for RS232 + 2 x DI + 2 x DO 1 x M12 for RS485 2 x Nano SIM Slot (4FF, Dual SIM Single Active) |  |
| Antenna Port | 4 x TNC connector for NR&LTE&GNSS antenna 2 x TNC connector for 2.4&5GHz Wi-Fi antenna |  |
| LED Indicators | 1 x Power indicator 1 x Network status indicator 1 x RSSI indicator |  |
| Button | 1 x reboot or restore button |  |
| MIMO | UL 2x2 MIMO on n38/n41/n48/n77/n78/n79 DL 4x4 MIMO on n1/n2/n3/n7/n25/n30/n38/n40/n41/n48/n66/n70/n77/n78/n79 and B1/B2/B3/B4/B7/B25/B30/B38/B40/B41/B42/B43/B48/B66 Wi-Fi 2x2 MIMO |  |
| Carrier Aggregation | LTE: DL 5CA and UL 2CA NR: 2CA |  |
| ENDC | Support |  |
| Power Supply | DC 12~36V |  |
| Power Consumption | < 24W |  |
| Dimensions | 259 x 178 x 48 mm |  |
| Weight | ≈2KG ¦ 4.41 lbs |  |
| Humidity | Operation/Storage: 5% - 95% (non-condensing) |  |
| Temperature | Operating temperature: -40 °C ~ +70 °C ¦ -40 °F ~ +158 °F Storage temperature: -40 °C ~ +85 °C ¦ -40 °F ~ +185 °F |  |

| SOFTWARE SPECIFICATIONS |  |  |
| --- | --- | --- |
| Data Services | 4 x APNs |  |
|  | Multiple PDN |  |
|  | IPV4/IPV6 compliance |  |
| LAN | IEEE 802.1Q VLAN |  |
|  | DHCP V4/V6 |  |
|  | DNS and DNS proxy |  |
|  | DMZ |  |
|  | UPNP |  |
|  | ALG |  |
|  | ARP/ICMP/IGMP |  |
|  | MAC address filtering |  |
| Management | Version management: Image checksum |  |
|  | HTTP/FTP auto-upgrade |  |
|  | TR069 | Upload/ Download the ACS specified file |
|  |  | Download the configuration file |
|  |  | Configure & queries parameters |
|  |  | Remote upgrade |
|  |  | Remote debugging operations |
|  |  | Cycle monitor |
|  | SNMP |  |
|  | WebGUI/ CLI | Various level of user login priority with different limitation and display |
|  |  | Encryption backup current settings and restore the backup settings |
|  |  | Export current diagnosis results and operation logs |
|  |  | Statistics of LAN link status, transmit and receive traffic and up time |
|  | Diagnostics |  |
|  | USIM PIN management and card authentication |  |
|  | USIM activation |  |
|  | NAT/Bridge mode |  |
|  | Port mirror/Port forwarding |  |
| VPN & Routing | Static Routing |  |
|  | IPSec |  |
|  | PPTP |  |
|  | GRE tunnel |  |
|  | L2TP V2/V3 |  |
| Security | Firewall |  |
|  | Mac address filtering |  |
|  | IP address filtering |  |
|  | URL filtering |  |
|  | Access control |  |
|  | HTTPS login from WAN |  |
|  | DDOS attack protection |  |
|  | Hierarchical user management |  |

| COMPLIANCE SPECIFICATIONS |  |
| --- | --- |
| Environment Compliance | Protection rating: EN60529:IP67 |
|  | Salt mist: IEC68-2-11,72H |
|  | Shock: IEC60068-2-27/EN50155/EN61373 |
|  | Free Fall: IEC60068-2-32 (1 meter height with package) |
|  | Vibration: IEC60068-2-6/EN50155/EN61373 |

Contact Us

www.btiwireless.com

sales@btiwireless.com